Supercapacitor

MoS2

- Chemically exfoliated-MoS2 (eMoS2) solution

  - Add 15 mg of MoS2 powder and add it to 30 mL DMF (or NMP) in a tube.

  - Block the top of it with the paraffin film.

  - Tip sonication (50 % power) for 1 h.

Plates in the images => exfoliated MoS2.

- Solvothermal-synthesized defect-free MoS2 (pMoS2)

  - Add ~1 mmol (~36 mg) hexaammonium heptamolybdate tetrahydrate ((NH4)6Mo7O24·4H2O, HHT, i.e. 7 mmol Mo) and ~14 mmol (~58 mg) thiourea in ~40 mL deionized water.

  - Stirring for 15 min (use magnetic stirring device).

  - Transfer the solution to Teflon-lined stainless steel autoclave (fasten everything tightly).

  - Incubating the solution at 220 C (overnight).

Thick MoS2 plates, yield = ~30 % of Mo source.

- Solvothermal-synthesized defect-rich MoS2 (nMoS2)

  - Add ~0.9 mmol (~30 mg) hexaammonium heptamolybdate tetrahydrate ((NH4)6Mo7O24·4H2O, HHT, i.e. 7 mmol Mo) and ~27 mmol (~100 mg) thiourea in ~40 mL deionized water.

  - Stirring for 15 min (use magnetic stirring device).

  - Transfer the solution to Teflon-lined stainless steel autoclave (fasten everything tightly).

  - Incubating the solution at 220 C (overnight).

Carbon nanotube (CNT)-ZIF (Zeolitic imidazole framework)-MoS2

- CNT

  - Add 5 mg of CNT powder in 30 mL DMF in a tube.

  - Add 30 mg of dopamine hydrochloride in the solution.

  - Block the top of it with the paraffin film.

  - Tip sonication (50 % power) for 30 min.

  - Stirring for 2 h (use magnetic stirring device).

    - Dopamine will be coated on CNT. Dopamine layer will act as a precursor for the growth of ZIF.

- CNT-ZIF

  - Add 30 mg of zinc chloride and 15 mg of 1h-1,2,4-triazole-3-thiol in the CNT solution

  - Block the top of it with the paraffin film.

  - Tip sonication (50 % power) for 10 min.

  - Incubating the solution for over 44 h at 120 C.

- CNT-ZIF-MoS2

  - CNT-ZIF-eMoS2

    - Add 10 mg of MoS2 in the CNT-ZIF solution

    - Tip sonication (50 % power) for 30 min.

    - Incubating the solution at 80 C (overnight).

  - CNT-ZIF-nMoS2

    - Vacuum-filtrate the CNT-ZIF solution. CNT-ZIF will form a dark film.

    - Wash the film with excess amount of EtOH.

    - Bake the film at 110 C (overnight).

    - Grind the film with glass stick and get CNT-ZIF powder.

    - Mix the powder with 0.9 mmol (~30 mg) hexaammonium heptamolybdate tetrahydrate ((NH4)6Mo7O24·4H2O, HHT, i.e. 7 mmol Mo) and 27 mmol (~100 mg) thiourea in ~40 mL deionized water.

    - Tip sonication (50 % power) for 30 min.

    - Transfer the solution to Teflon-lined stainless steel autoclave (fasten everything tightly).

    - Incubating the solution for 24 h at 220 C.

MoS2 nanoflowers are woven with CNT-ZIF network.

yield = 30~40 % of Mo source.

  - CNT-ZIF-pMoS2

    - Vacuum-filtrate the CNT-ZIF solution. CNT-ZIF will form a dark film.

    - Wash the film with excess amount of EtOH.

    - Bake the film at 110 C (overnight).

    - Grind the film with glass stick and get CNT-ZIF powder.

    - Mix the powder with ~1 mmol (~36 mg) hexaammonium heptamolybdate tetrahydrate ((NH4)6Mo7O24·4H2O, HHT, i.e. 7 mmol Mo) and ~14 mmol (~58 mg) thiourea in ~40 mL deionized water.

    - Tip sonication (50 % power) for 30 min.

    - Transfer the solution to Teflon-lined stainless steel autoclave (fasten everything tightly).

    - Incubating the solution for 24 h at 220 C.

CNT-ZIF fibers are completely packed with MoS2 plates.

MoS2 nanoplate, yield = 30~40 % of Mo source.